PROJECT AROC TECHNICAL LIBRARY Project AROC Book 7 - APRS Draft AROC-BOOK-0007 · REV 0.1 Build with Purpose. Document with Precision. Learn without End. James · KE4CON August 25, 2026

# Document Control

| Field | Value |
| --- | --- |
| Document Number | AROC-BOOK-0007 |
| Title | Book 7 - APRS |
| Revision | 0.1 |
| Status | Draft — 1 of many planned chapters |
| Owner | James / KE4CON |
| Technical Author | Claude (Claude Code) |
| Applies To | Project AROC Technical Library |
| License | CC BY-SA 4.0 (see LICENSE-CONTENT at repo root) |

# Revision History

| Rev | Date | Author | Description | Status |
| --- | --- | --- | --- | --- |
| 0.1 | 2026-08-25 | Claude (Claude Code) | First chapter — physical/link/application/network layers of APRS, grounded against aprs.org, TAPR, and APRS-Command's real implementation | Draft |

# Contents

| 1 | # What APRS Is, and How It Actually Works |
| --- | --- |

From a spoken sentence to a radio wave to a dot on a map — every layer, in order, with a real packet decoded byte by byte.

IN THIS CHAPTER

•  Why APRS was built for situational awareness, not vehicle tracking

•  The physical layer: how a 1200-baud tone pair becomes a bit

•  The link layer: the AX.25 frame, decoded field by field

•  The application layer: how a position report is written as text

•  The network layer: digipeating and the New-N Paradigm

•  How APRS-Command, real working software, implements every layer discussed here

# What APRS Is, and Why It Isn't 'Just Tracking'

APRS (Automatic Packet Reporting System) is a two-way, real-time digital communications system for amateur radio. Bob Bruninga, WB4APR, began developing the concept in the mid-1980s, with the system as it's known today taking shape around 1992. The common misconception is that APRS is a vehicle-tracking system — a ham-radio GPS breadcrumb trail. It isn't, and that distinction matters enough to build a whole chapter around.

JARGON, IN PLAIN WORDS Situational awareness means knowing what is happening around you, right now, without having to ask. A weather radar loop gives a meteorologist situational awareness. A dispatcher's map of every unit's location gives them situational awareness. That is what Bruninga actually built: a way for every station on a channel to maintain a shared, live picture of who and what is out there — positions, yes, but also weather, messages, objects, and resources — updated continuously and automatically, with no one having to call anyone else on voice to ask.

That distinction — situational awareness over tracking — is why APRS transmits redundantly at a decaying rate: a moving station beacons often while its position is changing quickly, then less often once it settles, so the network spends its limited airtime on what's actually new. A pure tracker would just log points; APRS is built to keep a shared picture current using the least channel time possible, because the channel is a shared, finite resource every station on frequency has to share.

The rest of this chapter follows one piece of information — a station's position — all the way down through every layer of the system: from the RF modulation that turns a bit into a tone, through the AX.25 frame that wraps it, to the plain-text format that encodes a latitude and longitude, to the digipeater network that relays it, to the Internet gateway that hands it to the rest of the world. By the end, a real packet is decoded byte by byte, and you'll be able to do the same to any packet you capture yourself.

# The Physical Layer: How a Byte Becomes a Radio Wave

## AFSK: two tones, not a shifting carrier

APRS over RF is carried on ordinary FM voice radios — the same rigs used for repeater chats — using AFSK (Audio Frequency-Shift Keying). The distinction in that name matters: in true FSK, the radio's carrier frequency itself shifts between two values. In AFSK, the carrier doesn't move at all; instead, two audio tones are fed into the microphone input (or a data port) and the radio's ordinary FM transmitter carries whichever tone is currently playing, exactly as it would carry a human voice. This is precisely why any FM handheld or mobile with a data/packet port — or even just a speaker-mic jack and a TNC — can transmit APRS: nothing about the radio itself needs to know APRS exists.

The specific tone pair is the Bell 202 standard, a modem tone scheme from 1970s telephone-line data communication that amateur packet radio adopted in the 1980s and never replaced: a 1200 Hz "mark" tone and a 2200 Hz "space" tone, switched at 1200 baud (1200 symbol changes per second — and since each symbol here is one bit, this is also 1200 bits per second).

## NRZI: why a steady tone means '1' and a changing tone means '0'

It would be simple to say "mark tone means bit value 1, space tone means bit value 0" — but that is not how AX.25 actually encodes bits onto those two tones. Instead it uses NRZI (Non-Return-to-Zero Inverted) encoding, and the rule inverts the intuition:

▪	A data bit of `1` is sent as no tone change — whichever tone was already playing keeps playing.

▪	A data bit of `0` is sent as a tone change — mark switches to space, or space switches to mark.

ENGINEERING NOTE Why encode it this way instead of the "obvious" direct mapping? NRZI is what makes bit stuffing (covered next) work at all, and bit stuffing is what makes the frame boundary marker (a fixed pattern of six 1-bits) unambiguous no matter what data is inside the frame. A long run of the *same* bit value produces a long run of tone *changes* under NRZI — which is exactly the behavior the receiver's clock-recovery circuit needs to stay synchronized, and exactly the property bit stuffing depends on to guarantee the flag pattern can never appear by accident inside real data.

| Bit stream | Tone sequence (NRZI) |
| --- | --- |
| 1 1 1 0 1 | mark, mark, mark, space, space |
| 0 0 1 1 0 | space, mark, mark, mark, space |

## Why 144.390 MHz — and why it's different elsewhere

In North America, essentially all APRS RF activity shares one nationally-coordinated frequency, which is what makes a single mobile digipeater or iGate useful to every APRS station that drives past it. Other ITU regions coordinated a different shared frequency for the same reason — there's nothing technically special about any of these numbers; the value is entirely in everyone agreeing to use the same one.

| Region | Frequency |
| --- | --- |
| North America | 144.390 MHz |
| Europe | 144.800 MHz |
| Australia | 145.175 MHz |

# The Link Layer: AX.25, Packet Radio's Envelope

Everything above this point has been about turning bits into tones. AX.25 ('Amateur X.25' — a link-layer protocol adapted from the ITU-T X.25 packet standard for ham radio) is what turns those bits into a structured, addressed, checksummed *frame* — an envelope with a "from," a "to," routing instructions, and a payload, the same conceptual job an Ethernet or Wi-Fi frame does for computer networks.

APRS uses exactly one AX.25 frame type: the connectionless UI (Unnumbered Information) frame. AX.25 also supports connection-oriented, numbered frames (for a reliable two-station link, like a BBS session) — APRS deliberately doesn't use any of that machinery, because a position report is a one-shot broadcast to everyone listening, not a private session with one other station.

| Field | Size | Contents |
| --- | --- | --- |
| Flag | 1 byte | 0x7E — marks the start of the frame |
| Destination address | 7 bytes | Callsign + SSID (conventionally APRS or a software-ID string, not a real station) |
| Source address | 7 bytes | The transmitting station's callsign + SSID |
| Digipeater address(es) | 0–56 bytes | Up to 8 relay-path entries, 7 bytes each |
| Control | 1 byte | 0x03 — UI frame |
| PID | 1 byte | 0xF0 — no layer-3 protocol |
| Information | up to 256 bytes | The actual APRS data — position, message, weather, etc. |
| FCS | 2 bytes | CRC-16 checksum, protecting everything from the destination address through the information field |
| Flag | 1 byte | 0x7E — marks the end of the frame |

## Callsigns packed into seven bytes

Each address — destination, source, or digipeater — is exactly 7 bytes: a 6-byte callsign field, then 1 SSID byte. The callsign is padded to 6 characters with spaces if shorter, and every character is left-shifted by one bit before being placed in the frame (this shift exists so the low bit of every callsign byte is free to be used as a flag elsewhere in the protocol). The 7th byte packs the SSID (Secondary Station Identifier, 0–15) — the number hams add after a callsign, like -9, to distinguish multiple stations or purposes under one license — together with two structural bits:

Bit: 7 6 5 4 3 2 1 0 Value: 1 1 1 S S S S L bits 7-5: always 1 1 1 (reserved) bits 4-1: SSID value, 0-15 bit 0 (L): end-of-address bit 0 = more addresses follow 1 = this is the last address in the frame

So a source address of `KE4CON-9` is packed as the shifted bytes for K, E, 4, C, O, N, followed by an SSID byte encoding the value 9 and the end-of-address bit — set to 1 only if no digipeater addresses follow it in the frame.

## Bit stuffing: keeping real data from impersonating a flag

The flag byte, 0x7E, is the bit pattern 01111110 — six consecutive 1 bits. If that exact six-1 run could ever occur naturally inside the frame's own data, a receiver would misread the middle of a frame as its end. AX.25 prevents this with bit stuffing: after transmitting five consecutive 1 bits anywhere between the opening and closing flags, the transmitter inserts an extra 0 bit that carries no data at all. The receiver does the reverse — after five consecutive 1 bits, it discards the next bit before it ever inserts a 0. The flag bytes themselves are exempt from this rule, which is exactly why a flag can never be confused with stuffed data: six-or-more consecutive 1 bits appears in the actual transmitted bitstream *only* at a flag.

## The information field and the checksum

The information field is where the actual APRS content lives — and unlike the address fields, its characters are plain, un-shifted ASCII. The FCS (Frame Check Sequence) is a 16-bit CRC (per ISO 3309), computed most-significant-bit-first and transmitted inverted, covering every byte from the destination address through the end of the information field. If a receiver recomputes the CRC over a received frame and it doesn't match the transmitted FCS, the frame is corrupted — most likely from another station transmitting at the same instant (a collision) — and is simply discarded. AX.25 has no retransmission mechanism for UI frames; a lost APRS packet is just gone, which is one more reason the whole system is built around frequent, redundant, decaying-rate updates rather than any single packet being guaranteed to arrive.

# The Application Layer: What's Actually Inside a Position Packet

## Position reports: turning a lat/lon into text a radio can send

The information field's first character is a Data Type Identifier that tells a receiving station what kind of APRS data follows — a position, a message, a weather report, an object, and more. Two of the most common are ! (a real-time position report, no timestamp) and @ (a position report with a timestamp, used when the report describes where a station *was* at a specific time rather than right now).

A real, complete uncompressed position report looks like this:

!4903.50N/07201.75W>Test1234

Decoded field by field:

| Characters | Meaning |
| --- | --- |
| ! | Data type identifier — real-time position, no timestamp |
| 4903.50N | Latitude: 49 degrees, 3.50 minutes North (the format is fixed-width ddmm.hh + N/S) |
| / | Symbol table identifier — / selects the primary symbol table |
| 07201.75W | Longitude: 072 degrees, 1.75 minutes West (fixed-width dddmm.hh + E/W) |
| > | Symbol code — on the primary table, > renders as a car icon on a map |
| Test1234 | Free-text comment — up to 40 characters, shown alongside the station on a map or in a station list |

## Symbol tables: one character chooses the icon

APRS defines two symbol tables — primary (selected with /) and alternate (selected with \) — each holding dozens of single-character icon codes (a car, a house, a digipeater, a weather station, and many more). The symbol table character and the symbol code character together are what a mapping application — like the one built into APRS-Command — uses to choose which icon to draw at a station's reported position.

SCOPE NOTE FOR THIS CHAPTER APRS also defines a compressed position format — a denser encoding (using a base-91 numbering scheme) that packs latitude, longitude, and optionally course/speed or altitude into fewer bytes than the fixed-width format shown above. It's real, it's in daily use, and it deserves its own full treatment — deliberately left to a later chapter rather than folded in here at reduced rigor.

# The Network Layer: How One Packet Reaches an Entire Region

## Digipeating: radios relaying radios

A single APRS station transmitting at typical handheld or mobile power might reach a few miles. To cover a region, APRS relies on digipeaters — stations, usually on hilltops or towers, that simply repeat any packet whose digipeater address field names them, then remove their own callsign from that field (or decrement a counter, covered next) so the packet isn't repeated by the same digipeater twice.

## The New-N Paradigm: WIDE2-2 and WIDE1-1,WIDE2-1

Early APRS used generic aliases like RELAY and WIDE, and operators would often chain several of them together (WIDE,WIDE,WIDE) hoping for wider reach — which instead flooded busy areas with duplicate packets as every digipeater in range repeated every hop. National operating rules were standardized around 2004 under the New-N Paradigm specifically to eliminate that inefficient routing.

The New-N Paradigm uses a generic alias with a built-in hop-count, written WIDEn-N — for example WIDE2-2 means "repeat this up to 2 more times, generically." Each time a digipeater repeats the packet, it decrements that trailing number by one; once it reaches zero, no further digipeater will repeat it. The current guidance:

▪	`WIDE2-2` — a general-purpose 2-hop path, the recommended default in essentially all areas for a station with no specific local knowledge of the digipeater network around it.

▪	`WIDE1-1,WIDE2-1` — a specialized 2-hop path for mobiles that have a nearby fill-in digipeater (a lower-power, local-coverage digipeater that only responds to WIDE1-1), letting that local digipeater pick the packet up first before it's repeated once more onto the wider regional network.

## APRS-IS: where RF meets the Internet

Digipeating alone only reaches as far as the RF network extends. APRS-IS (the APRS Internet System) is a worldwide, Internet-connected network of servers that any station can connect to directly, and that an iGate — a station configured to bridge the two worlds — feeds with everything it hears over RF. This is why a station transmitting nothing but a 5-watt handheld on a hilltop can still show up, moments later, on a map viewed from anywhere in the world: RF got the packet to one iGate, and APRS-IS carried it the rest of the way.

# Seeing It Work: APRS-Command as a Reference Implementation

Every layer described in this chapter is real, working code in APRS-Command, the cross-platform APRS client built as part of this project (GPL v3, C# / .NET, Avalonia UI — runs on Windows, macOS, Linux, and Raspberry Pi from one codebase). It's worth connecting the two directly, so this chapter isn't just theory:

| Layer in this chapter | Where it lives in APRS-Command |
| --- | --- |
| Link layer (AX.25 frames, packet parsing) | src/Aprs.Core — APRS packet types and parser |
| Physical/transport layer (serial KISS TNC, TCP KISS, AGWPE, APRS-IS) | src/Aprs.Transport |
| Digipeating (New-N Paradigm, fill-in and full modes) | src/Aprs.Services — configurable digipeater with alias support |
| APRS-IS / iGate bridging | src/Aprs.Services — iGate with packet-type filtering |
| Application layer (position reports, symbol tables) | Beacon scheduler + symbol picker in src/Aprs.Desktop, rendered via src/Aprs.Mapping |

The project's own README describes its guiding philosophy in terms that echo exactly what this chapter opened with: APRS-Command exists to give operators "a common operating picture" — situational awareness, not just a tracker — which is the same distinction Bob Bruninga built the whole protocol around forty years earlier.

# Why This Matters for Project AROC

An Amateur Radio Operations Center needs exactly what APRS was designed to provide: a shared, continuously-updating picture of positions, resources, and messages across everyone operating on a network — the same need behind Book 6 (Operations) and Book 10 (Emergency Communications). Understanding APRS from the tone pair up, not just as "the app that shows dots on a map," is what makes it possible to actually design, troubleshoot, and extend an operations center that depends on it — placing a digipeater, choosing a path, diagnosing why a station isn't showing up, or deciding whether a feature belongs in APRS-Command at all.

DESIGN TAKEAWAY Every layer in this chapter exists to solve one specific, real problem: AFSK/Bell 202 lets APRS ride on ordinary FM voice radios; NRZI and bit stuffing make the frame boundary unambiguous; AX.25's UI frame skips connection overhead a one-shot broadcast doesn't need; the New-N Paradigm exists because the old alternative flooded the airwaves; and APRS-IS exists because RF range alone was never going to be global. None of it is arbitrary — and that's the pattern worth carrying into every other chapter of this library.

# Sources & Further Reading

▪	aprs.org — Bob Bruninga's original APRS reference site: http://www.aprs.org/

▪	aprs.org — The New-N Paradigm: http://www.aprs.org/newN/new-N.html

▪	APRS Protocol Reference, Version 1.0.1 (APRS101.PDF): https://www.aprs.org/doc/APRS101.PDF

▪	Kenneth W. Finnegan, W6KWF — "Clarifying the Amateur Bell 202 Modem" (TAPR Digital Communications Conference 2014): https://files.tapr.org/meetings/DCC_2014/DCC2014-Amateur-Bell-202-Modem-W6KWF-and-Bridget-Benson.pdf

▪	Joshua Jerred — "APRS & AX.25 Demystified: Full Packet Generation and Frame Encoding": https://joshuajer.red/blog/2023-01-04-APRS-and-ax25

▪	APRS-Command source and documentation: https://github.com/KE4CON/APRS-Command